Dimensions in Inches,   Tolerances:  .XX  .03    .XXX   .010

DC CURRENT LEVEL DETECTOR

SERIES S58-800

#### CURRENT ACTUATION LEVELS

ZERO CURRENT INPUT	CONDITION A

200mA to 10A Max. 	CONDITION B

### OUTPUT

CONDITION A (ZERO CURRENT)	1K-Ohm max. between terminal 1 & 2 	CONDITION B (ABOVE 200mA)	40K-Ohm min. between terminal 1 & 2

EXCITATION VOLTAGE

VOLTAGE	28Vdc Nominal 22.8 to 30.4 Vdc min. to

max. voltage.

RESPONSE	1 m-sec. max.

ADJUSTMENTS	None

RESET FROM COND. B TO COND. A	Automatic when current levels falls within

0 to 200 mA.

##### ENVIRONMENTAL

TEMPERATURE RANGE	0C to 70C

Drawing Number	Rev.

700-S58-800	C

AAC

###### TERMINAL IDENTIFICATION

- +28Vdc EXCITATION VOLTAGE

- OUTPUT

- EXCITATION RETURN AND

OUTPUT RETURN (DC COMMON)

BARRIER BLOCK TERMINAL

6-23 TERMINAL SCREWS.